The design of a thermostat can be broken into three separate states off, heat, and cool. In the off state the system will remain idle, it will not activate either the heating circuit but will still communicate to them with the server updating the state and the temperature every thirty seconds.  During the heating the system will begin to monitor the temperature provided by the sensor when the temperature gets too low the thermostat system here will begin flashing the red LED but in an actual application it would send a signal to the heating unit to turn on to raise the temperature. The cooling would do this as well but instead of when warm it would turn on the cooling unit instead.  These all get cycled through when the button that controls the state is pressed.

To construct a thermostat, we need a few basic components, first we need a temperature sensor, a few buttons to control the system, and a display for the user to be able to get feedback from the unit. Other than that, there is a circuit board and a few LEDs to use during the prototyping.  The simplest peripherals are the buttons the buttons use General Purpose Input/Output to create interrupts that when pressed will either change the state or where the temperature is set.  GPIO is also implemented by the circuits since we only really need to tell the system if it should be on or off.  The system also uses Inter-Integrated Circuit Communication to communicate with the temperature/humidity sensor, here the Raspberry Pi acts as the controller and the sensor is a peripheral connected to it.  Universal Asynchronous Receiver-Transmitter (UART) is a serial communication protocol this is used to communicate over two wires this is used to transmit data to the server and for the display.

To prototype this thermostat, we started with a Raspberry Pi. This is a good choice for initial build, but it would not be the best case for a commercial product for consumers, as the costs tend to be more and power used, and reliability are a bit more. Raspberry Pi’s accel at prototyping, tinkering and for hobbyists.  The next choice we have is Freescale, these processors have adequate support for peripherals but may have a bit too much for a residential use case also lacks integrated Wi-Fi modules. This leads us to Microchip would be the best I feel that it has the right it has access to the peripherals we would need and as well as keep the cost at a reasonable price.

Reference

Best Microcontrollers – Deep Comparison Table 🔍 – Tricky circuit. (2025, March 11). https://trickycircuit.com/components/integrated-circuits/%F0%9F%94%9F-best-microcontrollers-deep-comparison-table-%F0%9F%94%8D/

GeeksforGeeks. (2023, November 2). Architecture of Raspberry Pi. GeeksforGeeks. https://www.geeksforgeeks.org/computer-organization-architecture/architecture-of-raspberry-pi/